Pre-Construction Safe Work Practices Certification

I, Employee Name, an employee of Contractor or Organization, certify that we will follow safe work practices on Address of Property.  Items 1A-1D will be adhered to, in compliance with Federal, state and local regulations, unless the work is exempt from safe work practice requirements as described in Item 2.

#### Check Number 1 or 2

- The following safe work practices will be applied as appropriate.

- The prohibited work methods listed below will not be used.

- Open flame burning or torching.

- Machine sanding or grinding without a high-efficiency particulate air (HEPA) local exhaust control.

- Abrasive blasting or sandblasting without HEPA local exhaust control.

- Heat guns operating above 1,100 degrees Fahrenheit, or those that that operate high enough to char the paint.

- Dry sanding or dry scraping. (For exceptions to this rule see 24CFR 35.140 (e).)

- Paint stripping in a poorly ventilated space using a volatile stripper that is a hazardous substance in accordance with regulations of the Consumer Product Safety Commission at 16 CFR 1500.3, and/or a hazardous chemical in accordance with the Occupational Safety and Health Administration at 29 CFR 1010.1200 or 1926.59, as applicable to the work.

- Protection of occupants and preparation of the worksite as described below.

- Occupant Protection

- Occupants shall not be permitted to enter the worksite during hazard reduction activities until final clearance is achieved.

- Occupants shall be temporarily relocated before and during hazard reduction activities if necessary.

- Dwelling unit and worksite shall be secured against unauthorized entry, and occupants’ belongings protected from contamination by dust-lead hazards and debris during hazard reduction activities.

- Occupants’ belongings in a containment area shall be relocated to a secure area outside the containment area or covered with appropriate materials.

- Worksite Preparation

- Worksite shall be prepared to prevent release of leaded dust and shall contain lead-based paint chips and other debris from hazard reduction activities within the worksite.

- A warning sign shall be posted at each entry to rooms where hazard reduction activities are conducted when occupants were present.

- Specialized cleaning after hazard reduction activities including:

- Use of HEPA vacuum cleaners; or other method of equivalent efficacy; and

- Lead-specific detergents or equivalents.

- Clearance testing of work site or unit will be conducted and achieved before re-occupancy shall be allowed.

- Safe work practices and clearance are not required when activities do not disturb painted surfaces below the de minimis thresholds defined below.

- The maintenance or rehab hazard reduction activities do not disturb painted surfaces that total more than:

- 20 square feet on exterior surfaces;

- 2 square feet in any one interior room or space; or

- 10 percent of the total surface area on an interior or exterior type of component with a small surface area (such as windowsills, baseboards, and trim).

- _____________________________	_____________             _____________________________	__________

- Contractor Signature			         	          Date 		City of       Representative	        	       Date